Bill of Quantities (BoQ) Form

(Refer e-procurement tender.bt for details and quoting)

| Sl. No. | BSR Code | Description of Works | UoM | Qty. | Rate (Nu.) | Amount | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A. Dismantling and Demolition Works |  |  |  |  |  |  |  |
| 1 | NA | Dismantling of toilet and bath room fittings such as european WC, Wash basin urinals mirrors, faucets, towel rails, paper holder, soap container etc. Fittings to be removed and stored properly for reuse. | Lumpsum | 2.00 |  |  |  |
| 2 | NA | Demolition & chippings of RCC floors, granite, floor and wall tiles, cement plaster, adhesives including disposal of rubbish all leads and lifts (As per instruction of engineering in-charge) | Lumpsum | 2.00 |  |  |  |
| 3 | NA | Demolishing of brick wall including disposal of rubbish within lead as per the site engineer. | Cu.m | 10.80 |  |  |  |
| 4 | NA | Taking out doors, windows and clerestory window shutter >3 sq.mm steel or wood including stacking within lead as per the instruction of engineering incharge. | each | 8.00 |  |  |  |
| B. Civil Works |  |  |  |  |  |  |  |
| 5 | NA | Providing & fixing Flush Door shutters (pre-fabricated), block-board core (including 4nos, 4" SS hing, 1no. 8" round door handle for both side of the door per door with screw) complete with all necessary accessories - 40mm | sq.m | 15.76 |  |  |  |
| 6 | NA | Providing & fixing 1mm sunmica (laminate) sheets pasted with specialized contact adhesives on flush door shutters back and front without any joint on the face, complete with all necessary accessories as required. | sq.m | 15.76 |  |  |  |
| 7 | NA | Providing & fixing in position, seasoned, dressed wood work in frames of doors, including fixing of anchore as instructed by site engineer, complete with all necessary accessories as required. - seasoned Class 'A' (conifer) | Cu.m | 0.32 |  |  |  |
| 8 | NA | Providing and laying in position plain cement concrete 1:1.5:3 (1 cement : 1.5 sand : 3 graded crushed rock 20 mm nominal size) excluding the cost of centering and shuttering. | Cu.m | 4.34 |  |  |  |
| 9 | NA | Providing the DR Fixet on RCC for flooring and cement plaster for walls as water proofing coatings. | Sq.m | 85.23 |  |  |  |
| 10 | NA | Providing & fixing antiskid tiles (600x600) mm in flooring, treads of steps and landings laid on bed of 12mm thick cement mortar 1:3 finished with flush pointing in white cement:- SOMANY/KAJARIA Brand | Sq.m | 33.69 |  |  |  |
| 11 | NA | Half - Brick Masonry (125mm)-Providing & laying Second-Class Half-brick Masonry (125 mm) in superstructure on 2nd floor level including cutting of wall for electrical pipes, sockets and switches- In cement mortar 1:3. | Sq.m | 58.05 |  |  |  |
| 12 | NA | Providing and fixing to frames 8mm PVC ceiling panel of approved texture, design and pattern including fixing of existing exhaust pipeline, cutting out holes for electrical fittings with all accessories as required. | Sq.m | 37.10 |  |  |  |
| 13 | FL0176 | Providing and laying Pre-Polished/ Polished Granite of approved quality and source in flooring using necessary cement bedding in C.M. 1:4 upto 20 mm thick average cement slurry for fixing the tiles with required slopes, curing, cleaning, filling the joints with pigments of similar colour and at all levels with all leads and lifts etc. complete as per directions of site in charge and as per pattern required. | Sq.m | 3.26 |  |  |  |
| 14 | NA | Providing & fixing toilet door lock with indicator Vacant/Engaged chrome latch deadbolt including all accessories as required. | each | 8.00 |  |  |  |
| 15 | NA | Providing & fixing 4"deadbolt on the top of toilet door complete including all accessories as required. | each | 8.00 |  |  |  |
| 16 | NA | Providing & fixing glazed tiles (300x300) mm in wall, with Cement or adhesive finished with flush pointing in white cement. | Sq.m | 186.54 |  |  |  |
| 17 | NA | Providing & fixing hydraulic door closer complete including all accessories as required.-DORMA TS72V BC door closer. | each | 2.00 |  |  |  |
| C. Plumbing and Sanitation Works |  |  |  |  |  |  |  |
| 18 | PO0370 | Providing & fixing P.V.C soil waste and vent pipes, single or double socketed, including pipe clip complete (excluding the cost of PVC fittings) - 75mm dia | m | 34.10 |  |  |  |
| 19 | PO0405 | Providing & fixing P.V.C plain bend - 75 mm Dia | each | 7.00 |  |  |  |
| 20 | PO0415 | Providing & fixing P.V.C door bend - 75 mm Dia | each | 8.00 |  |  |  |
| 21 | PO0435 | Providing & fixing Plain Tee - 75 mm Dia | each | 1.00 |  |  |  |
| 22 | PO0495 | Providing & fixing PVC plain double Y - 75 mm Dia | each | 1.00 |  |  |  |
| 23 | PO0565 | Providing & fixing P.V.C Nahani Trap - 110mm inlet & 75mm outlet. | each | 14.00 |  |  |  |
| 24 | PI0301 | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply, including all CPVC plain & brass threaded fittings, i/c fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & fittings with one step CPVC solvent cement and the cost of cutting chases and making good the same including testing of joints complete as per direction of Engineer in Charge. Concealed work, including cutting chases and making the good walls etc. - 20 mm nominal outer dia Pipes | m | 29.18 |  |  |  |
| 25 | PI0304 | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply, including all CPVC plain & brass threaded fittings, including fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & fittings with one step CPVC solvent cement and testing of joints complete as per direction of Engineer in Charge. Internal work - Exposed on wall - 40mm dia | m | 6.1 |  |  |  |
| 26 | NA | Providing and fixing white ceramic oval shape under counter wash basin including brackets.15mm c.p brass pillar taps, flexible stainless steel connecting pipe, 32 mm c.p brass waste coupling, 32mm dia c.p brass bottle trap and union complete with all accessories as required. and repair wall. | each | 6.00 |  |  |  |
| 27 | NA | Providing & fixing 40mm CPVC ball valve with all accessories as required. | each | 4.00 |  |  |  |
| 28 | NA | Providing & fixing European-type vitreous china wall hung w.c one piece Toilet with seat cover, repairing walls complete with all accessories as required. | each | 8.00 |  |  |  |
| 29 | NA | Providing & fixing white vitreous china flat back, lipped front urinal basin 430x260x350mm, with saperate brass infrared, battery powered auto flush valve for urinal, including all fittings, such as spreader, c.p connecting pipe, c.p brass bottle trap etc, with all accessories as required. | each | 4.00 |  |  |  |
| 30 | NA | Providing & fixing wall mounted Vitreous China Urinal Divider 630x150x320, Colour/Finish: Gloss White with all accessories as required. | each | 3.00 |  |  |  |
| 31 | PI0260 | Providing & fixing toilet paper holder- C.P. brass with all accessories as required. | each | 8.00 |  |  |  |
| 32 | PI0285 | Providing & fixing c.p coat hook- single with all accessories as required. | each | 8.00 |  |  |  |
| 33 | NA | Providing & fixing Liquid soap container, c.p, including c.p brass lid & brackets, c.p brass screws with all accessories as required. | each | 4.00 |  |  |  |
| 34 | PI0105 | Flush Valve & Bend- Providing & fixing c.p. flush valve, 32mm with 40mm outlet for toilet with all accessories as required. | each | 8.00 |  |  |  |
| 35 | NA | Providing & fixing touch free, 1150w hand drier including with all accessories as required. | each | 2.00 |  |  |  |
| 36 | NA | Providing & fixing C.P pvc health faucet including all accessories as required. | each | 8.00 |  |  |  |
| 37 | NA | Providing & fixing c.p. pvc stop cock- 15mm, angle, c.p knob with all accessories as required. | each | 20.00 |  |  |  |
| Grand Total |  |  |  |  |  |  |  |
|  | In word (Nu.): |  |  |  |  |  |  |

Note:

- Site Visit - Site survey and assessment of the Toilet (Compulsory) before submission of the bid.

- Must submit/upload rates in the Bills of Quantities (BoQ) Form as well as quote the rates in the e-procurement system.

- The Contractor shall bear all costs for safety equipment, PPE and related measures and shall be fully responsible for complying with all Occupational Health and Safety (OHS) requirements.

- No site camp or accommodation shall be provided by the Employer, and the Contractor shall make their own arrangements at their own cost for site camp, accommodation and related facilities.

Sign and Seal of the Bidder